📡

SFAS IoT Sensor Setup Guide

Complete Step-by-Step Configuration Manual

ESP8266 NodeMCU  ·  DHT22 Temperature & Humidity  ·  FC-28 Soil Moisture

Smart Farming Advisory System — Nyagatare District, Rwanda

| Hardware | ESP8266 NodeMCU v1.0 or v3 + DHT22 + FC-28 Soil Sensor |
| --- | --- |
| Firmware file | sfas_sensor.ino (upload with Arduino IDE) |
| Server file | modules/IoT/api/iotApi.php on your XAMPP |
| DB table | iot_readings + iot_devices (run iot_extension.sql) |
| Dashboard | modules/IoT/views/iot-dashboard.php |
| Route | /api/iot and /admin/iot-dashboard in index.php |

University of Kigali — BIT Capstone Project — June 2026

# 1. What You Have & What Each Does

You have three hardware components. Here is exactly what each one does:

| Component | Measures | How It Connects |
| --- | --- | --- |
| ESP8266 NodeMCU | — (it is the brain) | USB cable to your laptop. Runs the firmware and sends data over WiFi. |
| DHT22 Sensor | Temperature (°C) + Humidity (%) | 4 pins: VCC → 3.3V, DATA → D4, (skip NC pin), GND → GND |
| FC-28 Soil Moisture | Soil moisture 0–100% | 4 pins: VCC → 3.3V, GND → GND, AOUT → A0, DOUT → not used |

💡 Why these three sensors? Temperature and humidity from the DHT22 let the system detect conditions that favor fungal diseases (high humidity >85%) and heat stress (temperature >32°C). The soil moisture sensor tells the system when the soil is dry enough that irrigation is needed — the AI assistant uses this context when giving farming advice.

# 2. Wiring Diagram — Connect Everything

Complete connection table. Do this before installing any software. Use jumper wires.

## 2.1 DHT22 Sensor Connections

| DHT22 Pin | Label on sensor | Connect to NodeMCU | Note |
| --- | --- | --- | --- |
| Pin 1 (left) | VCC or + | 3.3V (NOT 5V) | Powers the sensor — must be 3.3V not 5V |
| Pin 2 | DATA or OUT | D4 (GPIO2) | Signal wire — put 10kΩ resistor between here and VCC |
| Pin 3 | NC | Leave unconnected | Not connected — skip this pin |
| Pin 4 (right) | GND or – | GND | Ground |

⚠️ Important: 10kΩ Pull-up Resistor Required The DHT22 DATA pin needs a 10kΩ resistor between it and VCC (3.3V). Without it you will get "nan" readings. Connect one leg to the DATA wire going to D4, other leg to the 3.3V wire. This is a small brown/orange component. If you do not have one, try the sensor anyway — some modules have it built in on the board.

## 2.2 Soil Moisture Sensor (FC-28) Connections

| FC-28 Pin | Label | Connect to NodeMCU | Note |
| --- | --- | --- | --- |
| Pin 1 | VCC | 3.3V | Power — use 3.3V for NodeMCU |
| Pin 2 | GND | GND | Ground |
| Pin 3 | AOUT | A0 | IMPORTANT: This is the one you need. Analog output. |
| Pin 4 | DOUT | Leave unconnected | Digital output — not used in SFAS |

⚠️ NodeMCU A0 Pin Limitation The NodeMCU ESP8266 has only ONE analog input pin — A0. This means you can only connect ONE analog sensor. The soil moisture AOUT goes here. If you want to add a second analog sensor later you need a multiplexer chip (CD4051 or similar).

## 2.3 Quick Reference — All Connections

| NodeMCU Pin | Connects To | What It Is |
| --- | --- | --- |
| 3.3V | DHT22 VCC + FC-28 VCC | Power supply for both sensors |
| GND | DHT22 GND + FC-28 GND | Ground for both sensors (share one GND pin) |
| D4 | DHT22 DATA | Temperature + Humidity signal |
| A0 | FC-28 AOUT | Soil moisture analog signal |
| USB (micro-USB) | Your laptop USB port | Powers the NodeMCU and uploads firmware |

# 3. Arduino IDE Setup (Do This Once)

Install the Arduino IDE and configure it for the ESP8266. Do this once — you never need to do it again.

| 1 | Download Arduino IDE Go to https://www.arduino.cc/en/software — Download the Windows installer. Install it with default options. |
| --- | --- |
| 2 | Add ESP8266 Board Support Open Arduino IDE → File → Preferences. In the "Additional Board Manager URLs" box, paste exactly: |
| 3 | Install ESP8266 Package Tools → Board → Boards Manager → search for "ESP8266" → click Install on the "esp8266 by ESP8266 Community" package. Wait for download to finish (~50MB). |
| 4 | Select the Right Board Tools → Board → ESP8266 Boards → select "NodeMCU 1.0 (ESP-12E Module)" |
| 5 | Select Upload Speed Tools → Upload Speed → 115200 |
| 6 | Install DHT22 Library Tools → Manage Libraries → search "DHT sensor library" → Install "DHT sensor library by Adafruit" → also install "Adafruit Unified Sensor" when asked |

ESP8266 board manager URL — copy this exactly:

https://arduino.esp8266.com/stable/package_esp8266com_index.json

💡 How to know it worked After installing the ESP8266 package, go to Tools → Board — you should see a new section called "ESP8266 Boards" with "NodeMCU 1.0 (ESP-12E Module)" in the list.

# 4. Calibrate the Soil Moisture Sensor (Do This First)

Every FC-28 sensor gives slightly different numbers. You need to find YOUR sensor's dry and wet values before the main code will give accurate percentage readings.

## 4.1 Upload the Calibration Sketch

- Connect the NodeMCU to your laptop via USB

- Open Arduino IDE → File → Open → navigate to sfas_calibrate.ino

- Verify the board is set to NodeMCU 1.0 (Tools → Board)

- Click the Upload button (right arrow →) and wait for "Done uploading"

- Open Serial Monitor: Tools → Serial Monitor

- Set baud rate to 115200 in the bottom-right dropdown of Serial Monitor

## 4.2 Find Your DRY Value

- With the soil sensor connected to A0, hold the sensor probe in DRY AIR — not touching soil, water, or anything

- Watch the Serial Monitor for about 10 seconds

- The numbers showing are your DRY readings — write down the highest stable number

Example: if you see 950, 948, 952, 949 → your SOIL_DRY_VALUE = 950

## 4.3 Find Your WET Value

- Fill a glass or bowl with plain water

- Dip the sensor probe (the two metal rods, NOT the electronics board) into the water

- Watch the Serial Monitor for about 10 seconds

- Write down the lowest stable number you see

Example: if you see 290, 285, 292, 288 → your SOIL_WET_VALUE = 285

## 4.4 Update the Main Sketch

Open sfas_sensor.ino and find these two lines near the top and replace with YOUR values:

const int SOIL_DRY_VALUE = 950; // ← replace 950 with YOUR dry value const int SOIL_WET_VALUE = 285; // ← replace 285 with YOUR wet value

💡 Typical FC-28 values Most FC-28 sensors read around 900–1023 when dry and 200–400 when wet. If your Serial Monitor shows 0 for soil, check that AOUT is connected to A0 (not DOUT). If it shows a constant 1023, the sensor may need 5V — try connecting VCC to the VU or VIN pin on NodeMCU (which outputs 5V from USB).

# 5. Configure the Main Sketch (sfas_sensor.ino)

Open sfas_sensor.ino in Arduino IDE. Find Section ① at the top — these are the ONLY lines you need to edit.

## 5.1 Find Your Computer's IP Address

The ESP8266 needs to know your XAMPP computer's IP address on the local WiFi network:

- On Windows: press Windows key + R → type cmd → press Enter → type: ipconfig → press Enter

- Look for "Wireless LAN adapter Wi-Fi" section → "IPv4 Address" → e.g. 192.168.1.105

- On the same WiFi: the ESP8266 and your XAMPP computer MUST be on the same WiFi network

⚠️ Same WiFi network required Your laptop (running XAMPP) and the NodeMCU MUST be on the same WiFi network. If your laptop is on university WiFi and your phone hotspot is different, it will not work. Use your phone hotspot for both, or connect laptop and ESP to the same home/office WiFi.

## 5.2 Edit These 5 Lines Only

// ════ EDIT THESE 5 LINES ════════════════════════════════════════ const char* WIFI_SSID = "YOUR_WIFI_NAME"; // ← your WiFi name const char* WIFI_PASS = "YOUR_WIFI_PASSWORD"; // ← your WiFi password const char* SERVER_IP = "192.168.1.105"; // ← YOUR PC IP from ipconfig const int SOIL_DRY_VALUE = 950; // ← from calibration Step 4.2 const int SOIL_WET_VALUE = 285; // ← from calibration Step 4.3 // Leave everything else unchanged

## 5.3 Also update your .env file on XAMPP

Open the .env file in your SFAS project root and add this line if it is not there:

IOT_SECRET=sfas_iot_secret_2026 # This must match the IOT_SECRET in sfas_sensor.ino exactly

## 5.4 Add new routes to index.php

Open your SFAS index.php router and add these two routes inside the $routes array:

'/api/iot' => 'modules/IoT/api/iotApi.php', '/admin/iot-dashboard' => 'modules/IoT/views/iot-dashboard.php',

## 5.5 Run the SQL extension

- Open phpMyAdmin → select sfas_db

- Click Import tab → choose iot_extension.sql → click Go

- You should see "Import has been successfully finished"

- Under Tables you will now see iot_readings and iot_devices

# 6. Upload the Firmware to the ESP8266

| 1 | Connect NodeMCU to laptop Use a micro-USB cable. Windows should make a sound when it connects. Check Device Manager — you should see a new COM port (e.g. COM5 or COM6). |
| --- | --- |
| 2 | Select the COM port Arduino IDE → Tools → Port → select the COM port that appeared (e.g. COM5). If you see multiple, unplug the NodeMCU and see which one disappears — that is the right one. |
| 3 | Open sfas_sensor.ino File → Open → select sfas_sensor.ino. Make sure you edited the 5 settings in Section 5.2 above. |
| 4 | Verify first (optional) Click the checkmark ✓ button (Verify). This compiles the code without uploading. If you see errors, read the error message — most common is a missing library. |
| 5 | Upload Click the right-arrow → button (Upload). The IDE will compile then upload. You will see progress dots in the status bar. NodeMCU LED will flicker. |
| 6 | Open Serial Monitor Tools → Serial Monitor. Set baud rate to 115200. You should see the boot messages within 5 seconds. |

💡 What success looks like in Serial Monitor You should see: "[WiFi] Connecting to: YourWiFiName" "[WiFi] ✓ Connected!" "[WiFi] IP Address: 192.168.1.108" "[SENSOR] Reading sensors..." " Temperature : 24.5 °C" " Humidity : 68.2 %" " Soil moisture: 42 %" "[HTTP] Response code: 200" "[HTTP] ✓ Data sent successfully!"

# 7. Verify Everything is Working

## 7.1 Check the Serial Monitor Output

| What you see | What it means |
| --- | --- |
| [WiFi] ✓ Connected! | WiFi working — good |
| Temperature: 24.5°C | DHT22 working — good |
| Soil moisture: 42% | Soil sensor working — good |
| Response code: 200 | SFAS received data — everything working! |
| [WiFi] TIMEOUT | Wrong WiFi name or password — check WIFI_SSID and WIFI_PASS |
| Temperature: nan | DHT22 not connected right — check wiring, add 10kΩ resistor |
| Soil raw: 0 | Soil sensor AOUT not on A0 — check wiring |
| Response code: 403 | Wrong IOT_SECRET — check .env matches sfas_sensor.ino |
| Connection FAILED | Wrong SERVER_IP — run ipconfig again and check |

## 7.2 Verify in phpMyAdmin

- Open phpMyAdmin → sfas_db → iot_readings table

- Click Browse — you should see rows appearing every 30 seconds

- If rows are appearing, the sensor is working end-to-end

## 7.3 Verify on SFAS Dashboard

- Log in to SFAS at http://localhost/sfas

- Click IoT Sensors in the navigation

- You should see the live green status dot and current readings

- The temperature and humidity numbers should match what Serial Monitor shows

# 8. Troubleshooting

## 8.1 DHT22 Problems

### Problem: Reads "nan" (not a number)

- Check VCC is connected to 3.3V (not 5V)

- Check DATA wire is on D4

- Add a 10kΩ pull-up resistor between DATA pin and VCC pin

- Try a fresh jumper wire on the DATA connection

- Wait 3 seconds after power-on before first read — DHT22 needs warm-up time

### Problem: Reads same value every time (stuck reading)

- Unplug and replug the USB cable to restart the NodeMCU

- Check that your jumper wires are firmly in the NodeMCU holes

## 8.2 Soil Moisture Sensor Problems

### Problem: Always reads 0%

- Check AOUT (not DOUT) is connected to A0

- Check VCC is connected — try 5V (VU or VIN pin) if 3.3V gives 0

### Problem: Reads 100% even in dry soil

- Your SOIL_DRY_VALUE is too low — redo the calibration with the sensor in dry air

- Check that GND is properly connected

### Problem: Percentage is negative or above 100%

- Your calibration values are swapped or wrong — redo calibration

- The code uses constrain() to clamp 0–100 so this should not show in the app

## 8.3 WiFi / HTTP Problems

### Problem: WiFi keeps timing out

- Check WIFI_SSID — WiFi names are case-sensitive ("MyWiFi" ≠ "mywifi")

- Check WIFI_PASS — look for spaces at the start/end, copy carefully

- Make sure your phone hotspot or router is on 2.4GHz — ESP8266 does NOT support 5GHz WiFi

- Move the NodeMCU closer to the router if signal is weak

### Problem: HTTP response code 403

- IOT_SECRET in sfas_sensor.ino does not match IOT_SECRET in .env

- Both must be exactly: sfas_iot_secret_2026 (or whatever you set)

- Restart Apache after changing .env

### Problem: Connection FAILED — cannot reach server

- Run ipconfig on your laptop and confirm the IP address

- Laptop and NodeMCU must be on the SAME WiFi network

- Apache must be running in XAMPP — check XAMPP Control Panel

- Try opening http://192.168.1.X/sfas in your phone browser (same network) — if it opens, the address is correct

- Disable Windows Firewall temporarily to test — if it works, add a firewall rule for port 80

### Problem: HTTP code 200 but no data in database

- Run iot_extension.sql in phpMyAdmin — the iot_readings table may not exist

- Check /api/iot route exists in index.php

- Check iotApi.php path is correct in index.php route

💡 Quick network test from your phone While the NodeMCU is trying to connect, open your phone browser (connected to the same WiFi) and go to http://192.168.1.X/sfas/api/iot?action=latest (use your PC's IP). If you see a JSON response, your server is reachable. If the page does not open, the IP is wrong or Apache is not running.

# 9. What to Say at Your Defense

When the panel asks about IoT, use this structured answer:

Panel Question: "How does your IoT integration work?" Your Answer: "SFAS uses an ESP8266 NodeMCU microcontroller connected to two sensors in the field: a DHT22 for air temperature and humidity, and an FC-28 capacitive soil moisture sensor. The node reads all sensors every 30 seconds and sends the data as a JSON HTTP POST to a dedicated REST API endpoint — /api/iot?action=ingest — in the SFAS backend. The data is stored in the iot_readings table in our MySQL database and displayed in real-time on the IoT Dashboard page, which auto-refreshes every 30 seconds." Panel Question: "Why IoT and not just weather API data?" Your Answer: "Open-Meteo gives regional weather data for the whole Nyagatare District. But microclimate conditions on a specific farm can be significantly different — a farm in a valley is cooler and more humid than one on a hillside 2km away. The IoT sensor captures hyperlocal data that the regional API cannot provide. The SFAS IoT Dashboard shows both readings side by side so farmers and agronomists can see the difference. Additionally, the soil moisture sensor provides data that no weather API offers — it tells us the actual water content in the soil, which is critical for irrigation decisions." Panel Question: "What happens if the sensor goes offline?" Your Answer: "The system handles this gracefully. The dashboard checks the timestamp of the last reading — if it is more than 2 minutes old, the status dot turns red and shows Offline. The rest of SFAS continues to function normally using the Open-Meteo weather API. The database stores all historical readings so when the sensor reconnects, the history chart picks up from where it left off. The firmware on the ESP8266 also has automatic WiFi reconnection logic and restarts itself if it fails 10 consecutive times."

# 10. Hardware Shopping List

Everything you need to buy in Kigali. All available at electronics shops in Nyarugenge (near UTC/Kigali City Tower area) or on Jumia Rwanda.

| Item | Where to buy | Approx cost (RWF) | Notes |
| --- | --- | --- | --- |
| ESP8266 NodeMCU v1.0 or v3 | Kigali electronics / Jumia | 8,000–12,000 | Any version works. v3 is newer. |
| DHT22 temperature & humidity sensor | Kigali electronics / Jumia | 2,000–4,000 | Sometimes sold as AM2302. |
| FC-28 soil moisture sensor module | Kigali electronics / Jumia | 1,500–3,000 | Buy the module (with board), not bare probes. |
| Jumper wires (male-to-male + male-to-female) | Kigali electronics | 500–1,000 | Pack of 40 is more than enough. |
| 10kΩ resistor (for DHT22 pull-up) | Kigali electronics | 100–200 | Buy a pack of 10 — very cheap. |
| Micro-USB cable | Any phone shop | 1,000–2,000 | For uploading firmware and power. |
| Breadboard (optional, makes wiring easier) | Kigali electronics | 1,500–2,500 | Strongly recommended for demo. |
| TOTAL |  | ~15,000–25,000 RWF |  |

💡 Buying tip If you cannot find the parts in Kigali in time, you can also order from Jumia Kenya (jumia.co.ke) with 2–3 day delivery. Search: "ESP8266 NodeMCU" and "DHT22 module" and "soil moisture sensor module FC-28". Make sure to order the MODULE versions which include the PCB board, not just bare components.

SFAS IoT Setup Guide — Version 1.0 — June 2026

ESP8266 + DHT22 + FC-28 Soil Sensor  ·  University of Kigali  ·  Nyagatare District